# Plan and Design

# My Project

- My project investigates the effects of soil moisture, light intensity, temperature and rainfall on drought conditions.

- I considered focusing on wildfire risk, as wildfires are influenced by soil moisture, which I identified as a key factor through my research. However, I opted for drought as it is directly linked to severe wildfires, plant growth and forest health. Droughts are also an early warning sign for wildfires.

# Design Objectives

- My micro:bit will collect environmental data on soil moisture, light intensity and temperature; these will be my analogue inputs. I decided against measuring other variables such as sound or movement because they are not relevant to drought modelling. I will use the ‘datalogger’ extension to store data, as it is simpler to use than serial. The system will start automatically when button A is pressed and record environmental readings every 30 minutes until button A is pressed again to stop it. This satisfies br1 and br2.

- When the micro:bit detects abnormal conditions such as high temperature and low soil moisture, it will simulate a drought cycle in a forest and display an alert icon on its LED matrix to notify the user that there is a drought. I considered using a string, but the display is too small to read easily, so an icon is more effective. I can manually adjust soil or temperature conditions to see how the system responds to a changing environment. This satisfies br1, br3 and ar3.

- I will develop a Python model using a CDI-inspired drought index to calculate drought severity from collected data combined with open-source precipitation data. CDI (Combined Drought Index) is commonly used by the EDO (European Drought Observatory), combines multiple environmental indicators to determine drought severity. However, implementing the full CDI would be too complex for this project, so I decided on a simplified weighted index model based on similar principles. In my model, temperature and light intensity data are used to calculate evapotranspiration as it increases water loss thus increasing drought conditions, which is then combined with soil moisture and open-source precipitation data to calculate drought severity. This allows my model to output drought levels and test “what-if” scenarios, based on different inputs, such as increased temperatures or decreased rainfall. This satisfies ar1 and ar2.

- I will extend the system by increasing logging frequency as drought severity rises and activating a sound alert when conditions become severe. I considered making an automatic water trigger, but I rejected it due to its complexity and time constraints. This satisfies ar3.

- Stakeholders and End Users

- My end users are nature conservationists and forestry companies, who need regular monitoring of drought conditions in a forest. Stakeholders include anyone who lives near forests, as drought increases wildfire risk and can lead to environmental and property damage.

- My system must accurately assess drought conditions using data collected and stored on the system and communicate changes clearly to users.

- Technologies Used

- I will be using a micro:bit V2 to collect temperature and light data with its sensors.

- Soil moisture will be measured using an external analogue probe made using nails connected through the micro:bit pins.

- The micro:bit will run MakeCode programs that collect and output data.

- Data will be stored on the device using the ‘datalogger’ extension and exported as a CSV file.

- I will use Python to build a CDI-inspired drought index to calculate drought severity, using the pandas module to store and manipulate data.

- Flowchart